Download Arduino IDE and flash from there directly with the code below – can also convert into .uf2 file and then you can drag the file onto the pico after pressing BOOTSEL (quicker once the code is how you want it).

#include <OneWire.h>

#include <DallasTemperature.h>

// ---------------------- Pins ----------------------

#define ONE_WIRE_BUS 7

#define MOSFET_PIN   28

// ---------------------- Sensor ----------------------

OneWire oneWire(ONE_WIRE_BUS);

DallasTemperature sensors(&oneWire);

// ---------------------- Temperature control ----------------------

double Input = 0.0;

// Hysteresis thresholds

const double HEATER_ON_TEMP  = 32.75;

const double HEATER_OFF_TEMP = 33.25;

// Track current heater state

bool heaterOn = false;

// ---------------------- Timing ----------------------

unsigned long lastTempRead = 0;

const unsigned long tempInterval = 500;  // ms

void setup() {

Serial.begin(115200);

delay(2000);

pinMode(MOSFET_PIN, OUTPUT);

digitalWrite(MOSFET_PIN, LOW);   // heater off for safety

sensors.begin();

sensors.setResolution(11);       // 0.125 C steps

Serial.println("Starting DS18B20...");

Serial.print("Number of sensors found: ");

Serial.println(sensors.getDeviceCount());

}

void loop() {

unsigned long now = millis();

// ---------------------- Read temperature ----------------------

if (now - lastTempRead >= tempInterval) {

lastTempRead = now;

sensors.requestTemperatures();

float tempC = sensors.getTempCByIndex(0);

if (tempC == DEVICE_DISCONNECTED_C) {

heaterOn = false;

digitalWrite(MOSFET_PIN, LOW);

Serial.println("Error: DS18B20 disconnected. Heater OFF.");

return;

}

Input = tempC;

// ---------------------- Hysteresis control ----------------------

if (Input <= HEATER_ON_TEMP) {

heaterOn = true;

}

else if (Input >= HEATER_OFF_TEMP) {

heaterOn = false;

}

digitalWrite(MOSFET_PIN, heaterOn ? HIGH : LOW);

}

// ---------------------- Serial debug ----------------------

static unsigned long lastPrint = 0;

if (now - lastPrint >= 1000) {

lastPrint = now;

Serial.print("Temp: ");

Serial.print(Input, 3);

Serial.print(" C | Heater ON at <= ");

Serial.print(HEATER_ON_TEMP, 2);

Serial.print(" C | Heater OFF at >= ");

Serial.print(HEATER_OFF_TEMP, 2);

Serial.print(" C | Heater: ");

Serial.println(heaterOn ? "ON" : "OFF");

}

}